14550 E. Sands Ranch Rd.                   Vail, AZ 85641                    (520) 647-3760

Fire Inspection

General

__Yes 	 __No 	__NA	Home has smoke alarms on every level.

__Yes   __No __NA	Home has a smoke alarm in every bedroom.

__Yes   __No __NA	Smoke alarms are located outside each separate sleeping area.

__Yes   __No __NA	Smoke alarms are located at least 10 feet from a stationary or fixed                  			cooking appliance.

__Yes   __No __NA	For larger homes (where the interior floor area on a given level is greater than 1,000     square feet), there is an average of at least 1 smoke alarm for every 500 square feet.

__Yes   __No __NA	Smoke alarms are interconnected so when one sounds, they all sound. (Best protection)

__Yes   __No __NA	Home has ionization smoke alarms.

__Yes   __No __NA	Home has photoelectric smoke alarms.

__Yes   __No __NA	Home has combination (photoelectric and ionization) smoke alarm

__Yes   __No __NA	No  All smoke alarms are working.

__Yes   __No __NA	Family has a home fire escape plan.

__Yes   __No __NA 	Family practices the home fire escape plan at least twice a year.

__Yes   __No __NA 	The home has occupant(s) that require assistance to escape. Occupants discuss escape planning and occupant requirements in case of a fire or emergency escape.

__Yes   __No __NA 	House number is visible from the street.

__Yes   __No __NA	Windows used for escape open easily — not blocked by furniture, security bars or     nailed/painted shut.

__Yes   __No __NA 	Security bars, if present, have a quick release-device.

__Yes   __No __NA 	Home has a home fire sprinkler system. (Best protection)

__Yes   __No __NA 	Home has a carbon monoxide alarm outside each separate sleeping a

__Yes   __No __NA 	Home has a carbon monoxide alarm on every level.

__Yes   __No __NA 	Carbon monoxide alarms are working throughout the home.

__Yes   __No __NA 	Smokers smoke outside the home.

__Yes   __No __NA	Large ashtrays are provided outside for smokers.

__Yes   __No __NA 	Matches and lighters are stored in a secure cabinet.

__Yes   __No __NA	Lit candles are not left unattended.

__Yes   __No __NA 	Flammable liquids, if stored in the home, are limited in quantify.

__Yes   __No __NA 	The home is not cluttered with clothes, magazines, newspapers and other items that can burn.

Kitchen

__Yes   __No __NA 	Things that can burn are removed from the stovetop.

__Yes   __No __NA 	Small appliances are unplugged when not in use.

__Yes   __No __NA	 Adult stays in the kitchen when stovetop is in use.

Living/Family Room

__Yes   __No __NA 	Fireplace has proper screen and hearth.

__Yes   __No __NA 	Things that can burn are at least 3 feet from space heaters and fireplaces.

Bedrooms

__Yes   __No 	__NA	 All bedrooms have two ways out — window and a door.

Garage

__Yes   __No __NA 	There is a solid door between garage and residence.

__Yes   __No	__NA	Gas powered equipment is stored in an outside shed or garage, separate                                    		from the home.

__Yes   __No __NA 	Gasoline is stored in an approved safety container in an outside shed or                                                                           		garage, separate from the home.

Outside the Home

__Yes   __No __NA	Outside electrical receptacles are GFCI and they are in good working                                             		condition.

__Yes   __No __NA	There is no rubbish, trash, brush or tree trimmings accumulation on the                                    		property.

__Yes   __No __NA	Barbecue grill is only used outdoors.

__Yes   __No __NA	Swimming pool or hot tub is enclosed by a four-sided fence and locked.

Electrical

__Yes   __No __NA 	AFCIs (arc-fault circuit interrupters) are installed throughout the home                                         			and they are working properly (new home construction only).

__Yes   __No __NA 	Kitchen and bathrooms have GFCI outlets on countertop surfaces within 6                      		feet of running water outlets and they are working properly.

__Yes   __No __NA 	All receptacle and switch faceplates are installed and in good condition.

__Yes   __No __NA	Receptacles have been tested and are in good working condition — no      			evidence of arcing or overheating.

__Yes   __No __NA	Switches are in good condition — no evidence of arcing or overheating.

__Yes   __No __NA	Lighting fixture canopies are fastened in place and fixture is in good  			condition.

__Yes   __No __NA 	Bulbs in light fixtures are the correct wattage for the lighting fixture.

__Yes   __No  __NA	Flexible cords and cables are not used as fixed wiring, run through holes                 		in walls, ceiling or floor, run through doorways or windows or under   			carpets or attached to building surfaces.

__Yes   __No __NA 	If there are young children in the home, the home has tamper-resistant         		outlets.

__Yes   __No __NA 	Panel board and distribution equipment is accessible for inspection and in        		good condition — no evidence of overheating, corrosion or other damage.

__Yes   __No __NA  	Service entrance raceways or cables are fastened in place, grounded,      			readily accessible and in good condition.

Stair Safety Trips and falls on stairs are a leading cause of home injuries.

__Yes   __No __NA 	Stair is provided with a handrail(s).

__Yes   __No __NA	Stair can be illuminated for night time use.

__Yes   __No __NA	Stair tread depth and riser height are uniform.

__Yes   __No  __NA	Stair landing nosing projections are consistent with other nosings on the              		stair flight.     (You can determine this by doing a “crouch and sight” test.       		Crouch down at the top landing     of the stair. All the nosings from the 		            landing to the bottom step should line up.)

With your consent, the Home Fire Safety Survey Team has done a fire inspection of your home. The items checked “No” may put you and your family at risk. You are urged to correct these at once for your own safety. This inspection does not identify future conditions such as failure of components, wiring or human behavior which could result in a fire.

Home Fire Safety Survey

Home Address:  _____________________________________________________________________

WAIVER, RELEASE AND HOLD HARMLESS AGREEMENT

In consideration of the voluntary performance of a home fire safety inspection  and/or smoke alarm or battery installation of my home located at _______________________________________________, I, on behalf of myself, and all members of family, as well as my heirs, executors, administrators or successors, hereby waive any claim or cause of action of any nature that I have, or in the future may have, against any and all individual or organizational participants in the Home Fire Safety Survey Program, including but not limited to the Rincon Valley Fire District, and its officers, agents or employees, which claim or cause of action grows out of or results from a fire or other damage, following the said home fire safety inspection; and I further hereby agree to release and hold harmless any and all organizational and individual participants including the aforesaid fire department and municipality in the Home Fire Safety Survey Program from and against all damages of any kind, to persons or property, growing out of or resulting from a fire in my referenced home.

I acknowledge having read, understood, and agreed to the above waiver, and release.

________________________________________________________ Print name Signature Date

________________________________________________ Witness (print name) Signature Date

*This form generally indicates that the occupant or owner of the property agrees to waive his or her rights to sue any individual, any municipality and any other organizations or individuals involved in the fire safety inspection of this home, if a fire occurs after the inspection. The purpose of the waiver is to protect the individual or any of the organizations involved against liability arising from the home fire inspection. This statement is intended for information only, the terms of the waiver themselves shall prevail if there are any questions. You should seek advice if you do not understand this waiver.